12-State Wireless E911 ASR order guide for: - T1 facility out of Access Tariff - Type 2C Wireless 911 trunks

T1 ASR (stand-alone T1 ASR order): - Requested from Access Tariff - Switch Access or Special Access (Muxed) - Dedicated for 911 services - Channelized (at DS1 handoff)

| Key ASR Fields | Value/Comments |
| --- | --- |
| Proj | WENSA911 |
| REQTYP | SD |
| PIU | 100 |
| LTP [applicable to Switch Access only, not applicable to Special Access] | Applicable LTP value: - End Office = HA (Riding T3 CFA- CFA field populated) - End Office = GF (No T3 CFA- SCFA field is blank) |
| SPEC | - SW and MW Region = E911FAC - West Region = CELLS |
| REMARKS | Should indicate the following as examples: T1 for 911 Type 2C trunks @ ODDSTXEM3ED T1 for 911 Type 2C trunks @ HLWDCA01CT9 T1 for 911 Type 2C trunks @ BLISILBIDS0 NOTE: If WSP is not requesting diverse T1 facilities for 911, please note in Remarks as an example- ‘Electing not to req diversity frm AT&T’ |
| NC | Applicable NC value, refer to Carrier Coding Guides: https://clec.att.com/clec/hb/shell.cfm?section=2237&hb=1151&redirectsection=2788 - SW and MW Region: T1s terminate directly to a switch (i.e. HCED,HC-D, HCEH, HC-H) - West Region: T1 terminate at a MUXLOC/Hub (i.e. HCEM, HC-M) NOTE: T1s in West Region connect to a Hub/Mux location, refer to additional info below under SECLOC NOTE: 3rd character of NC = ‘E’ (B8ZS/T1ZF) 3rd character of NC = ‘-‘ (SF/AMI) |
| NCI | Applicable NCI value, refer to Carrier Coding Guides: https://clec.att.com/clec/hb/shell.cfm?section=2237&hb=1151&redirectsection=2788 NOTE: NCI of the higher level facility the T1 will ride |
| SECLOC | - SW and MW Region: Applicable 11 character CLLI code of the AT&T Selective Router NOTE: T1 for 911 is a connection directly to the 911 Selective Router switch, AT&T will design to the Master Location of the 911 Selective Router switch, will be provided on FOC [Firm Order Confirmation]. - West Region: Applicable 11 or 8 character CLLI code of the 911 MUXLOC/Hub NOTE: T1 for 911 is a connection to the 911 Hub/Mux locations, refer to the W – Selective Router Table document (columns ‘J’ and ‘K’) posted to Prime Access: https://primeaccess.att.com/shell.cfm?section=4931 Examples: SECLOC = BKFDCA12K02 SECLOC = AUBNCA01 |

Additional T1 information: - Once FOC is received and RID has passed on the ‘Access T1’ ASR, the ‘Type 2C Trunk-only’ ASR can be submitted. - On FOC the T1 CFA/ECCKT should have an “E” in facility designation, as example: 101E/T1ZF (identifies Emergency Services) - NOTE: These guidelines are for only the T1 facility that will support Type 2C 911 trunks (DS0s) at an AT&T 911 Selective Router/SECLOC (i.e. HLWDCA01CT9, FTWOTXED1ED, ROCHMIMNDS0). They are NOT for meet-point billed (MPB) T1s that may be used for 911 services at a non-AT&T switch/central office (i.e., BRSWCAXH1ED). Those ASRs should be treated BAU (business as usual) for a MPB T1 facility out of Access Tariff between AT&T and another ILEC, because the MPB T1 is not used for 911 svcs at an AT&T Selective Router/SECLOC (i.e., Facility ACTL in AT&T territory and SECLOC in ILEC territory.)

Type 2C 911 Trunk ASR (trunk only ASR): - Must have T1 CFA prior to Trunk ASR issuance - WSP obtains the dESN assignment from the applicable E911 PSAP/Customer - May also refer to the following Prime Access web site for ASR Examples: https://primeaccess.att.com/asr/ - State = Select a State - Product Classification = “M” Wireless Trunking - Product = Wireless Trunking, select Search

| Key ASR Fields | Value/Comments |
| --- | --- |
| Proj | WENSA911 |
| REQTYP | MD |
| WST | L (Type 2C 911 trunks) |
| LTP | FF (trunks only) |
| RPON | If diversity (T1 facilities) requested populate this field – example: PH ASR relates the AF ASR, and vice versa (refer to TGTYP field below) |
| NC | - SDBE (SS7 signaling) - SDCE (MF Signaling) |
| NCI | Applicable NCI value, refer to Carrier Coding Guides: https://clec.att.com/clec/hb/shell.cfm?section=2237&hb=1151&redirectsection=2788 NOTE: NCI of the higher level facility the T1 will ride |
| CFA | T1 CFA obtained on FOC from the stand-alone T1 ASR |
| TRFTYP | E9 (911 trunks) |
| PSAP | “E” + PSAP/E911 Customer’s Name Examples: EWalker Co SO, Los Angeles CHP |
| ESN | 5 numeric default ESN (dESN) provided by the PSAP/E911 Customer Examples: 00125 |
| SECLOC | AT&T 911 Selective Router CLLI |
| TGTYP | - SW Region = E (Direct Final (DF) to End Office) - MW Region = E (Direct Final (DF) to End Office) F (Direct Final (DF) to Tandem) - West Region: - Option One Diversity = A (Primary High (PH) to End Office), H (Alternate Final (AF) to End Office) - Option Two Diversity = E (Direct Final (DF to End Office) |
| APON | Used for West Region Option One Diversity: only populate this field on the PH trunk group ASR with the PON# of the related AF trunk group ASR |
| DIR | 1T (one-way terminating) |
| CPN | Y (required for SS7 signaling) |

*Note: This document is intended to be a guide only. For a complete list of requirements, refer to the Access Service Ordering Guidelines (ASOG) and Carrier Coding Guide (CCG).