# ASR Information

The following are ASR examples for ordering AT&T Dedicated Ethernet Service.  The examples do not necessarily represent all possible configurations, or the ASR fields required to order AT&T Dedicated Ethernet Service.  Please refer to the Access Service Ordering Guideline (ASOG) for additional ASR details.

AT&T Dedicated Ethernet (ADE) ASR Job Aid

AT&T Dedicated Ethernet ASR can be recognized by the following fields:

Required ASR Field Data for REQTYP “SD”

| ASR Field/Form | Data |
| --- | --- |
| ACTL | Must be populated with (11) character CLLI |
| CNO | For ASR 1st, this field is left blank. Otherwise, Pre-Sales ID# provided by AT&T Account Team: ADXXXXXXX (9 character CNO beginning with “AD”) |
| SPEC Code | ADENET or ADEOOF (Out-of-franchise) ADEOOF is never used for ASR 1st |
| VTA | Must be 12, 24, 36 or 60 (if other VTA, PNUM must be present) |
| PNUM | Broadband (BSA) Contract # (if applicable) |
| SR | Special Routing – Diversity or Protection request (if applicable) This field is never used for ASR 1st |
| DIVPON | Diverse PON – required with Port or IWC Diversity (if applicable) This field is never used for ASR 1st |
| NC | Type of Network Channel from TAB 16 ADE section of the CCG – See Tables 1 & 2 below. |
| NCI | Network interface at ACTL/POP/A location - See Tables 1 & 2 below |
| SECNCI | Network interface at SECLOC/Customer Premise/Z location - See Tables 1 & 2 below |
| SECLOC | Secondary locations – populate with E and use the SALI (Service Address Location Information) for address details. |
| SPOT | Populated with CLLI (XPOI) of SECLOC address (optional) |
| CFA | If higher speed aggregation is requested (if applicable) – See Note 1 below |
| SCFA | If higher speed aggregation is requested (if applicable) – See Note 1 below |

Note 1: ADE does not allow BOTH OTN CFA and SCFA on a single circuit.